GENERAL SAFETY

- Use a chair with high seat and arms to help you sit and stand up with ease.

- Remove obstacles such as loose rugs, draught excluders and worn floor coverings.

- Ensure lighting in hallways and stairways is adequate.

- Wear flat supportive shoes.

- Avoid wet floors.

- Beware of outdoor hazards including uneven surfaces, wet leaves and ice.

- Do not store this product in temperatures below freezing.

- Take special care if carrying a bag or similar package.

- All equipment should be regularly inspected for any damage or significant signs of wear.

- Regular cleaning with a mild disinfectant is recommended. DO NOT use bleach or any solvents (such as lighter fluid).

Check regularly for:

- Bent or damaged tubes or cracks at the tube joints

- Loose or worn horseshoe clips or both buttons of the spring clips are present.

- Loose screws or fasteners.

- Elongated adjustment holes.

- Loose handles.

- Loose or worn wheels and fasteners (where fitted)

- Smooth, worn or split ferrules (rubber tips).

Replace all worn, missing or damaged parts where necessary.

Model Nos: SMM001-SMM010

| Maximum User Weight: | 160kg (25st) (Do not exceed this weight) |
| --- | --- |

WARNING

For safety, these precautions must be taken when using these products. Before each use make sure:

- Walker is adjusted to correct height.

- Locking mechanisms and spring buttons are fully engaged.

- Walker sits level and all legs are adjusted to correctly.

- Ferrules are in good condition - replace if necessary.

- Do not exceed the maximum weight limit for the frame.

- Use only Simplymed Ltd parts or accessories.

- When using accessories such as bags, baskets etc, ensure these are fitted safely and are not overloaded.

- Stow the frame with care when transporting in a car boot.
Damage can occur due to impact with car boot door.

USING THE STANDARD WALKING FRAME

Please read all instructions before use.

PLEASE NOTE:

These are our recommendations but should be checked with your physiotherapist.

When the frame is adjusted correctly the user should be able to maintain an upright posture with the elbow slightly flexed. In this way, body weight is taken through the frame by pushing down on the frame when walking.

CONTINUED